# Using Raspberry Pi 5 with Laptop over USB-C

This guide explains how to configure a Raspberry Pi 5 and control it directly from a laptop using a single USB-C cable. It covers enabling the necessary configuration, connecting the Pi, and controlling it through the laptop's terminal.

# 1. Preparing the Raspberry Pi

1. Insert the microSD card with Raspberry Pi OS into your Raspberry Pi 5.

2. Boot the Pi with a monitor, keyboard, and mouse connected.

3. Open a terminal and edit the configuration file:

- sudo nano /boot/firmware/config.txt

4. Add the following line at the end of the file:

- dtoverlay=dwc2,dr_mode=peripheral

5. Save and exit (CTRL+O, ENTER, CTRL+X).

6. Next, edit the command line configuration:

- sudo nano /boot/firmware/cmdline.txt

- Insert after rootwait (all on one line):

- modules-load=dwc2,g_ether

- Example:

- console=serial0,115200 console=tty1 root=PARTUUID=40d75242-02 rootfstype=ext4 fsck.repair=yes rootwait modules-load=dwc2,g_ether quiet splash plymouth.ignore-serial-consoles cfg80211.ieee80211_regdom=PK

⚠️ Keep it all on one line.

7.Enable SSH (if not already)

- sudo raspi-config

- Interface Options → SSH → Enable

8. Save and exit. Reboot the Pi:

sudo reboot

# 2. Connecting Raspberry Pi to Laptop

1. Power off the Pi.

2. Connect the Raspberry Pi 5 to your laptop using a USB-C.

3. Power on the Raspberry Pi.

4. On your laptop, a new serial device will appear.

# 3. Accessing Raspberry Pi from Laptop

Windows Users:

- Connect via SSH

- ssh serenity@raspberrypi.local

- Username: serenity

- Password: serenity

If .local doesn’t work → use the IP assigned to the USB interface.

4. Controlling the Raspberry Pi

Once connected, you will see the Raspberry Pi login prompt. Enter:

Username: pi
   Password: raspberry (default, unless changed)

Now you can use the Pi directly from your laptop terminal.

Examples:

ls            # list files

pwd           # print working directory

whoami        # check logged-in user

uname -a      # system info

# 5. Running Python Environment (TinyBERT)

1. Update and upgrade system:

sudo apt update && sudo apt upgrade -y

2. Create a Python virtual environment:

python3 -m venv tinybert-env

3. Activate the environment:

source tinybert-env/bin/activate

4. Install dependencies inside the environment (e.g., PyTorch, Transformers).

# 6. Limitations & Next Steps

- Using USB-C serial gives you terminal-only access.
- For graphical desktop, enable SSH + X11 forwarding or VNC via 'sudo raspi-config'.
- To transfer files, use 'scp' over SSH or plug in a USB drive.

# 7. How to get Graphical Desktop for Raspberry Pi 5 on your laptop

1️⃣ Install VNC Viewer on your laptop

- Go to RealVNC Viewer Download

- Choose your OS (Windows, macOS, Linux) → Install

2️⃣ Enable VNC Server on Raspberry Pi

- On your Pi terminal:

- sudo raspi-config
Interface Options → VNC → Enable

- Exit raspi-config and reboot if prompted:

- sudo reboot

After this, the Pi is ready to accept VNC connections.

3️⃣ Find your Pi’s IP

You already have it:

ifconfig

- wlan0 IP = 192.168.100.39

4️⃣ Connect using VNC Viewer

- Open VNC Viewer on your laptop.

- In the VNC Server field, enter your Pi’s IP:

192.168.100.39

- Click Connect → It will ask for credentials:

  - Username: serenity

  - Password: serenity

5️⃣ Start using the Pi Desktop

- After successful login, you’ll see the Raspberry Pi GUI on your laptop.

- You can now control the Pi with mouse and keyboard, just like it’s plugged into a monitor.