Full Guide: Raspberry Pi → Windows Audio Streaming

Requirements

- Raspberry Pi 5 running Raspberry Pi OS (with desktop).

- Windows laptop (any version) with speaker output.

- USB-C cable (or LAN connection) to connect Pi to Windows.

- PulseAudio for Windows: Download link.

Step 1: Install PulseAudio on Windows

- Download and extract PulseAudio:

- C:\Users\tahaa\Downloads\pulseaudio-1.1

- Create a simple config folder:

- C:\Users\tahaa\pulseaudio-config

- Create default.pa in that folder with contents:

# Minimal PulseAudio config for Windows

set-default-sink wave_output

load-module module-native-protocol-tcp auth-anonymous=1

load-module module-waveout

load-module module-null-sink

- Test PulseAudio manually:

- >C:\Users\tahaa\Downloads\pulseaudio-1.1\bin\pulseaudio.exe -n -F C:\Users\tahaa\Downloads\pulseaudio-1.1\pulseaudio-config\default.da

- Warnings about “secure directory creation” are normal.

- Audio is now listening on TCP port 4713.

- Optional: create a Windows shortcut to auto-start PulseAudio with the config:

start /B C:\Users\<YourUser>\Downloads\pulseaudio-1.1\bin\pulseaudio.exe -n -F C:\Users\<YourUser>\pulseaudio-config\default.pa

Step 2: Allow TCP Port Through Windows Firewall

- Open Windows Defender Firewall → Advanced Settings → Inbound Rules.

- Add a new TCP rule:

  - Port: 4713

  - Allow connection for your current network

  - Name it: PulseAudio TCP

Step 3: Configure Raspberry Pi

- Check current audio server:

pactl info

- Modern Pi OS uses PipeWire.

- Set Windows IP as PulseAudio server:

export PULSE_SERVER=<Windows-IP>

Replace <Windows-IP> with your Windows laptop IP (LAN or USB-C adapter).

- Test with:

paplay /usr/share/sounds/alsa/Front_Center.wav

- You should hear the sound through Windows speakers.

If you cannot hear the sound, run this in raspberry pi terminal:

- pactl set-default-sink wave_output

- systemctl --user restart pipewire pipewire-pulse

Step 4: Make Pi Audio Routing Permanent

- Create PulseAudio client config:

- mkdir -p ~/.config/pulse

- nano ~/.config/pulse/client.conf

- Add:

- default-server = tcp:<Windows-IP>

- Restart audio services:

- systemctl --user restart pipewire pipewire-pulse

- Now all Pi audio automatically goes to Windows.

Step 5: Optional – Use with VNC

- Open RealVNC Viewer to control the Pi.

- Audio plays on Windows while you control the Pi remotely.

Step 6: Troubleshooting

| Issue | Fix |
| --- | --- |
| No sound | Check $PULSE_SERVER is correct. |
| Firewall blocks | Allow TCP port 4713 inbound. |
| Still null sink | Ensure Windows PulseAudio default sink is wave_output. |
| Latency or drop | Use LAN instead of USB-C adapter if needed. |

Step 7: Automate Everything

- On Windows: use a shortcut in Startup to auto-start PulseAudio.

- On Pi: $PULSE_SERVER is set in ~/.config/pulse/client.conf, so all audio automatically goes to Windows on boot.

Result:

- Raspberry Pi 5 audio now plays through your Windows laptop speakers.

- Works with VNC or any Pi application (media, TTS, system sounds).

- Fully automated with persistent configuration.